GVWR-

GVWR:                                           Trailer to meet 68,000 Pound GVWR
VIN PLATE:                                    US ­ VIN Plate to Read in English (Standard)
Product Type:                               Dry Van Trailer

Geometry-

*  Length - Exterior:                     48'­0"
** Width - Interior:                      101"
Height - Exterior:                          13'­6"
Height - Interior:                           110" Nominal
Deck Height:                                  Standard application
Fifth Wheel Height:                      47"

Tires & Wheels-

Tire Size:                                          295/75R22.5
Wheel Material:                             All Steel Wheel or Aluminum
Wheel Size:                                     8.25 x 22.5, 5­Hand Hole (Steel, Inner & Outer)

Floor-
Floor Specification: Floor              1.38" (1­3/8") Nominal Domestic Laminated Oak Hardwood
Ratings - VT: Floor-                         18,000 lbs. Rated Floor per TTMA
Assembly Method:                         Wood Floor Assembly

L/Gear Crank Handle: Roadside

Running Gear-
Axle Spacing:                                   49" Axle Spacing
Axle ABS System:                            Tandem Axle  ABS System
Option­Related Bogie Stopper Feature:  STD plug stopper
Suspension Capacity:                     Standard Duty
Type of Susp Spring:    	                Air Bag
Spring Specification:                       Air Bag

Rear-

* Rear Frame:                                  Roll­Up Door, 6" Roll­Up Header (New Design)
Rear Door Opening Spec.:             Standard Application
Floor Level:                                      Door to be Flush with Floor (STD)

Roof-

Roof Panel:                                       0,040" Aluminum  (Translucent Top) Preferred

*Install E-Track on Inside Walls for Cargo Tie Down